Work Sheet 2

NAME: MOHAMED MIGDHADH

STUDENT ID: S2200065 (VILLA COLLEGE) | 22023480 (UWE)

MODULE CODE: UFCEVK-15-2

MODULE TITLE: INTERNET OF THINGS

Introduction:

The task of this work sheet was to Implement a simple communication protocol that transmits data between 2 BBC Micro:bit devices, using GPIOs. 	For the protocol I have chosen morse code. And the code was written in Micro:bit python editor. Once the code is flashed to the Micro:bit it will act as a transceiver (both transmitter and received)

Design steps:

- Set up all the necessary modules as shown below.

- The alphabet and number storage, as well as the corresponding morse code storage are declared as shown below.

- “microbit.display.show(alphabet_numbers[alph_num_index])” is used to display the current character to be sent.

- “if microbit.button_a.is_pressed() ==1:” this code is used to check if the button A is pressed and if true increment the index by one. If index is out of bounds, reset it to 0.

- “if microbit.button_b.is_pressed() ==1:” is used to check if the button B is pressed and if true “radio.send(morse_code[alph_num_index])” will broad cast the corresponding morse code string to all other devices in the group.

- “incoming = radio.receive()” this code allows  the other devices to grab the incoming broadcast messages.

- Once the incoming message is received the string of morse code is scrolled on the led screen of the Micro:bit using “microbit.display.scroll(incoming)”

Inputs and their corresponding outputs:

- Button A is pressed.

(Before A is pressed)

(After A is pressed)

- Button B is pressed.

(Before B is pressed)

(After B is pressed)